// Math 24 for Arduino

// Instead of RPN， resolving by removing parenthesis， * & /， then + -， as human does.

// Alvin Chan， Project Assoicate， PolyU HK

// 2018-12-12 Initial version

#include <Keypad.h>

#include <Wire.h>               // Wire Library for I2C

#include <LiquidCrystal_I2C.h>  // NewLiquidCrystal Library for I2C

// Define LCD pinout

const int  en = 2， rw = 1， rs = 0， d4 = 4， d5 = 5， d6 = 6， d7 = 7， bl = 3;

// Define I2C Address - change if required

const int i2c_addr = 0x3F;

LiquidCrystal_I2C lcd(i2c_addr， en， rw， rs， d4， d5， d6， d7， bl， POSITIVE);

String title = "Math24: Calc 24 with 4 no. & ()+-*/";

String author = "by Alvin Chan， CPCE， HKPU";

const byte ROWS = 4; //four rows

const byte COLS = 3; //three columns

char keys[ROWS][COLS] = {

{'1'，'2'，'3'}，

{'4'，'5'，'6'}，

{'7'，'8'，'9'}，

{'*'，'0'，'#'}

};

byte rowPins[ROWS] = {8，7，6，5}; //connect to the row pinouts of the keypad

byte colPins[COLS] = {4，3，2};   //connect to the column pinouts of the keypad

Keypad keypad = Keypad( makeKeymap(keys)， rowPins， colPins， ROWS， COLS );

String Goal = "24";

String Q[] = {"Q1: 2，2，6，8 ?"，	    // (8-2-2)*6

"Q2: 2，2，4，5 ?"，	    // 4*5+2+2

"Q3: 2，4，4，8 ?"};	    // (8-4)*(4-2)

int q = 0;    // Next question no.

String Space = "                ";    //16 spaces to clear the 2nd row

void setup () {

lcd.begin(16，2);      // Set display type as 16 char， 2 rows

lcd.setCursor(0，1);   // Print on 2nd row 1st

lcd.print(author); a

lcd.setCursor(0，0);   // Print on 1st row 2nd

lcd.print(title);

// This sequence repositions the cursor at the end of Row 0

delay(2000);

lcd.autoscroll();     // Scroll the rest to see

for (int thisChar = 16; thisChar < title.length(); thisChar++) {

// title.length() is a method of the String object to get the length

// So loop through the end of the string

lcd.print(title.substring(thisChar，thisChar+1));

// title.substring(from， to) is to get the char to print 1-by-1

delay(500);

}

lcd.noAutoscroll();   // Stop the scrolling

delay(2000);

lcd.clear();

// Flash backlight 2 times

for(int i = 0; i< 2; i++){

lcd.backlight();

delay(250);

lcd.noBacklight();

delay(250);

}

lcd.backlight();        // Turn backlight back on

}

void loop () {

String Answer = "";

char key = "";

int operatorSequence = 0;

int answerLength = 0;

lcd.setCursor(0，0);     // Line 1 display

lcd.print(Q[q]);

lcd.setCursor(0，1);     // Line 2 display

lcd.print("=              ");

if (q<2){q++;}else{q=0;}	// Rotate the 3 questions

do {

key = keypad.getKey();

if(key) {

answerLength= Answer.length();

switch (key){

case '#': break;	// Enter key

case '0': 		// BackSpace key

if (answerLength!=0){Answer.remove(answerLength-1);}

operatorSequence = 0;		// Reset the Operator key

break;

case '*':   // To insert parenthesis after an operator， enter a number and BS first

switch (operatorSequence){

case 0: Answer+='('; break;

case 1: Answer.remove(answerLength-1); Answer+=')'; break;

case 2: Answer.remove(answerLength-1); Answer+='+'; break;

case 3: Answer.remove(answerLength-1); Answer+='-'; break;

case 4: Answer.remove(answerLength-1); Answer+='*'; break;

case 5: Answer.remove(answerLength-1); Answer+='/'; break;

}

if (operatorSequence <6) {operatorSequence++;}

else {operatorSequence =0;}

break;

default:   // '1'， '2'，'3'，'4'，'5'，'6'，'7'，'8'，'9':

Answer += key; 			// Append the new entry

operatorSequence = 0;

break;

}

if (key != '#'){

lcd.setCursor(2，1);

lcd.print(Space);		// Clear the space after =

lcd.setCursor(2，1);

lcd.print(Answer);      // Refresh the Answer

}

}

} while (key != '#');

// Here is the location to add syntax error checking

Answer = parsing(Answer);

if (isDigit(Answer[0])){

if (Answer == Goal){Answer= "Correct";}

else{Answer= Answer +" is incorrect";}

lcd.setCursor(0，1);

lcd.print(Space);

lcd.setCursor(0，1);

lcd.print(Answer);

delay(2000);

}else{

lcd.setCursor(0，1);

lcd.print(Space);

lcd.setCursor(0，1);

lcd.print("Please Reset!");

delay(10000);

}

}

String parsing(String Operation){

int operationLength = Operation.length();

int operationLastIndex = operationLength -1;

int optrPost= -1;  // differentiate from any index

String Prefix = "";

String Suffix = "";

String Result = "";

for (int h=0; h<=operationLength; h++){ // Remove parenthesis

switch (Operation[h]){

case '(': optrPost = h; break;  // Target the last left parenthesis

case ')':                       // Encounter the 1st right parenthesis

if (optrPost > 0) {

Prefix = Operation.substring(0， optrPost);

}

if (h != operationLastIndex){

Suffix = Operation.substring(h+1);

}

Result = parsing(Operation.substring(optrPost+1， h));

if (Prefix =="" && Suffix ==""){

return Result;

}else{

for (int i=1; i<Result.length(); i++){

if (Result[i]== '*' || Result[i]== '/' || Result[i]== '+' || Result[i]== '/'){

Result = parsing(Result);		// In case there are more than 1 operator

break;

}

}

return parsing(Prefix + Result + Suffix);	// Remove any more parenthesis

}

break;

}

}

for (int j=0; j<=operationLength; j++){   		// No parenthesis

switch (Operation[j]){    // Solve * & / in this round

case '*': return processing(j， Operation); break;

case '/': return processing(j， Operation); break;

}

}

for (int k=0; k<=operationLength; k++){   // Solve for + & - in this round

switch (Operation[k]){

case '+': return processing(k， Operation); break;

case '-': return processing(k， Operation); break;

}

}

}

String processing(int index， String Optn){

String Pfx = "";

String Sfx = "";

String Operand1 = "";

String Operand2 = "";

if (index<3){       //From beginning， 1 or 2 digits

Operand1 = Optn.substring(0， index);

} else { if (isDigit(Optn [index-2])) {   //2 digits

Operand1 = Optn.substring(index-2，index);

} else {          //1 digit

Operand1 = Optn[index-1];

}

Pfx = Optn.substring(0，index-2);

}

if ((Optn.length() -1-index) < 3) {   //Till ending， 1 or 2 digits

Operand2 = Optn.substring(index+1);

} else { if (isDigit(Optn[index+2])) {    //2digits

Operand2 = Optn.substring(index+1，index+3);

Sfx = Optn.substring(index+3);

} else {          //1 digit

Operand2 = Optn[index+1];

Sfx = Optn.substring(index+2);

}

}

if (Pfx =="" && Sfx ==""){

return evaluate(Operand1， Operand2， Optn[index]);

}else{

return parsing(Pfx + evaluate(Operand1， Operand2， Optn[index]) + Sfx);

}

}

String evaluate(String Operand1， String Operand2， char optr){

int digit1= Operand1.toInt();

int digit2= Operand2.toInt();

switch (optr){

case '+': return String(digit1 + digit2); break;

case '-': return String(digit1 - digit2); break;

case '*': return String(digit1 * digit2); break;

case '/': return String(digit1 / digit2); break;

}

}