/*

@file SEN0233.ino

@brief Air Quality Monitor (PM 2.5, HCHO, Temperature & Humidity)

@n Get the module here: https://www.dfrobot.com/product-1612.html

@n This example is to detect formaldehyde, PM2.5, temperature and humidity in the environment.

@copyright  [DFRobot](http://www.dfrobot.com), 2017

@copyright GNU Lesser General Public License

@author [lijun](ju.li@dfrobot.com)

@version  V1.0

@date  2017-03-01

*/

#include <SoftwareSerial.h>

SoftwareSerial PMSSerial(3, 4); // Arduino board Software RX, TX

char col;

unsigned int PMSa = 0,FMHDSa = 0,TPSa = 0,HDSa = 0,PMSb = 0,FMHDSb = 0,TPSb = 0,HDSb = 0;

unsigned int PMS = 0,FMHDS = 0,TPS = 0,HDS = 0,CR1 = 0,CR2 = 0;

unsigned char buffer_RTT[40]={};     //Serial buffer; Received Data

char tempStr[15];

void setup()

{

Serial.begin(115200);

PMSSerial.begin(9600);

}

void loop()

{

while(!PMSSerial.available());        //Wait until PMSSerial is available

while(PMSSerial.available()>0)        //Data check: weather there is any Data in PMSSerial

{

for(int i=0;i<40;i++)

{

col =PMSSerial.read();

buffer_RTT[i]=(char)col;

delay(2);

}

PMSSerial.flush();

CR1 =(buffer_RTT[38]<<8) + buffer_RTT[39];

CR2 = 0;

for(int i=0;i<38;i++)

CR2 += buffer_RTT[i];

if(CR1 == CR2) {                   //Check data integrity

PMSa=buffer_RTT[12];         //Read PM2.5 High 8-bit

PMSb=buffer_RTT[13];         //Read PM2.5 Low 8-bit

PMS=(PMSa<<8)+PMSb;          //PM2.5 value

FMHDSa=buffer_RTT[28];       //Read Formaldehyde High 8-bit

FMHDSb=buffer_RTT[29];       //Read Formaldehyde Low 8-bit

FMHDS=(FMHDSa<<8)+FMHDSb;    //Formaldehyde value

TPSa=buffer_RTT[30];         //Read Temperature High 8-bit

TPSb=buffer_RTT[31];         //Read Temperature Low 8-bit

TPS=(TPSa<<8)+TPSb;          //Temperature value

HDSa=buffer_RTT[32];         //Read Humidity High 8-bit

HDSb=buffer_RTT[33];         //Read Humidity Low 8-bit

HDS=(HDSa<<8)+HDSb;          //Humidity value

}

else {

PMS = 0;

FMHDS = 0;

TPS = 0;

HDS = 0;

}

}

Serial.println("-----------------------uart--------------------------");

Serial.print("Temp : ");

sprintf(tempStr,"%d%d.%d",TPS/100,(TPS/10)%10,TPS%10);

Serial.print(tempStr);

Serial.println(" C");           //Serial print Temperature

Serial.print("RH   : ");

sprintf(tempStr,"%d%d.%d",HDS/100,(HDS/10)%10,HDS%10);

Serial.print(tempStr);          //Serial print humidity

Serial.println(" %");

Serial.print("HCHO : ");

Serial.print(FMHDS);

Serial.println(" ug/m3");       //Serial print formaldehyde, unit: ug/m³

Serial.print("PM2.5: ");

Serial.print(PMS);

Serial.println(" ug/m3");       //Serial print PM2.5, unit: ug/m³

Serial.println();

}